# SECTION 1: Identification of the substance/mixture and of the company/undertaking

## Product identifier

Identification of the substance	Coco amidopropyl betaine

CAS number	78930-61-6

Alternative name(s)	1-Propanaminium, 3-amino-N-(carboxymethyl)-N,N-dimethyl-, N-coco acyl derivs., hydroxides, in-ner salts

## Relevant identified uses of the substance or mixture and uses advised against

Relevant identified uses	General use

## Details of the supplier of the safety data sheet

Formulator County

2300 kathryn in plano tx 75025

e-mail: sales@formulatorcounty.com

## Emergency telephone number

| Poison centre |  |  |  |  |
| --- | --- | --- | --- | --- |
| Country | Name | Postal code/city | Telephone | Telefax |
| United Kingdom | National Poison Information Centre Medical Toxicology Unit | SE14 5ER Lon- don | +44 171 635 91 91 |  |

# SECTION 2: Hazards identification

## Classification of the substance or mixture

### Classification acc. to GHS

| Section | Hazard class | Category | Hazard class and cat-egory | Hazard state-ment |
| --- | --- | --- | --- | --- |
| 3.2 | skin corrosion/irritation | 2 | Skin Irrit. 2 | H315 |
| 3.3 | serious eye damage/eye irritation | 2 | Eye Irrit. 2 | H319 |
| 3.4S | skin sensitisation | 1 | Skin Sens. 1 | H317 |
| 4.1C | hazardous to the aquatic environment - chronic hazard | 3 | Aquatic Chronic 3 | H412 |

For full text of abbreviations: see SECTION 16.

### The most important adverse physicochemical, human health and environmental effects

Spillage and fire water can cause pollution of watercourses.

## Label elements

Labelling

    - Signal word	warning

    - Pictograms

GHS07

### Hazard statements

H315	Causes skin irritation.

H317	May cause an allergic skin reaction.

H319	Causes serious eye irritation.

H412	Harmful to aquatic life with long lasting effects.

### Precautionary statements

P261	Avoid breathing dust/fume/gas/mist/vapours/spray.

P273	Avoid release to the environment.

P280	Wear protective gloves/protective clothing/eye protection/face protection. P333+P313	If skin irritation or rash occurs: Get medical advice/attention.

P337+P313	If eye irritation persists: Get medical advice/attention. P362+P364	Take off contaminated clothing and wash it before reuse. P501	Dispose of contents/container to industrial combustion plant.

## Other hazards

### Results of PBT and vPvB assessment

According to the results of its assessment, this substance is not a PBT or a vPvB.

SECTION 3: Composition/information on ingredients

## Substances

Name of substance	Coco amidopropyl betaine Identifiers

CAS No	78930-61-6

EC No	263-058-8

Molecular formula	C19H38N2O3

Molar mass	342.5 g/mol

SECTION 4: First aid measures

## Description of first aid measures

### General notes

Do not leave affected person unattended. Remove victim out of the danger area. Keep affected person warm, still and covered. Take off immediately all contaminated clothing. In all cases of doubt, or when symptoms persist, seek medic-al advice. In case of unconsciousness place person in the recovery position. Never give anything by mouth.

### Following inhalation

If breathing is irregular or stopped, immediately seek medical assistance and start first aid actions. In case of respirat-ory tract irritation, consult a physician. Provide fresh air.

### Following skin contact

Rinse skin with water/shower.

### Following eye contact

Remove contact lenses, if present and easy to do. Continue rinsing. Irrigate copiously with clean, fresh water for at least 10 minutes, holding the eyelids apart.

### Following ingestion

Rinse mouth with water (only if the person is conscious). Do NOT induce vomiting.

## Most important symptoms and effects, both acute and delayed

Symptoms and effects are not known to date.

## Indication of any immediate medical attention and special treatment needed

none

SECTION 5: Firefighting measures

## Extinguishing media

### Suitable extinguishing media

Water, Foam, Alcohol resistant foam, ABC-powder

### Unsuitable extinguishing media

Water jet

## Special hazards arising from the substance or mixture

Deposited combustible dust has considerable explosion potential.

### Hazardous combustion products

Nitrogen oxides (NOx), Carbon monoxide (CO), Carbon dioxide (CO2)

## Advice for firefighters

In case of fire and/or explosion do not breathe fumes. Co-ordinate firefighting measures to the fire surroundings. Do not allow firefighting water to enter drains or water courses. Collect contaminated firefighting water separately. Fight fire with normal precautions from a reasonable distance.

SECTION 6: Accidental release measures

## Personal precautions, protective equipment and emergency procedures

### For non-emergency personnel

Remove persons to safety.

### For emergency responders

Wear breathing apparatus if exposed to vapours/dust/spray/gases.

## Environmental precautions

Keep away from drains, surface and ground water. Retain contaminated washing water and dispose of it. If substance has entered a water course or sewer, inform the responsible authority.

## Methods and material for containment and cleaning up

### Advice on how to contain a spill

Covering of drains, Take up mechanically

### Advice on how to clean up a spill

Take up mechanically.

### Other information relating to spills and releases

Place in appropriate containers for disposal. Ventilate affected area.

## Reference to other sections

Hazardous combustion products: see section 5. Personal protective equipment: see section 8. Incompatible materials: see section 10. Disposal considerations: see section 13.

# SECTION 7: Handling and storage

## Precautions for safe handling

### Recommendations

    - Measures to prevent fire as well as aerosol and dust generation

Use local and general ventilation. Take precautionary measures against static discharge. Use only in well-ventilated areas. Ground/bond container and receiving equipment.

### Specific notes/details

Dust deposits may accumulate on all deposition surfaces in a technical room. The product in the delivered form is not dust explosion capable; the enrichment of fine dust however leads to the danger of dust explosion.

### Advice on general occupational hygiene

Wash hands after use. Do not eat, drink and smoke in work areas. Remove contaminated clothing and protective equipment before entering eating areas. Never keep food or drink in the vicinity of chemicals. Never place chemicals in containers that are normally used for food or drink. Keep away from food, drink and animal feedingstuffs.

## Conditions for safe storage, including any incompatibilities

### Managing of associated risks

    - Explosive atmospheres

Removal of dust deposits.

### Ventilation requirements

Use local and general ventilation.

## Specific end use(s)

See section 16 for a general overview.

# SECTION 8: Exposure controls/personal protection

## Control parameters

| Occupational exposure limit values (Workplace Exposure Limits) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coun-try | Name of agent | CAS No | Identi-fier | TWA [ppm] | TWA [mg/mS] | STEL [ppm] | STEL [mg/mS] | Ceiling-C [ppm] | Ceiling-C [mg/mS] | Nota-tion | Source |
| GB | dust |  | WEL |  | 10 |  |  |  |  | i | EH40/20 05 |
| GB | dust |  | WEL |  | 4 |  |  |  |  | r | EH40/20 05 |

Notation

Ceiling-C	ceiling value is a limit value above which exposure should not occur i	inhalable fraction

r	respirable fraction

STEL	short-term exposure limit: a limit value above which exposure should not occur and which is related to a 15-minute peri-od (unless otherwise specified)

TWA	time-weighted average (long-term exposure limit): measured or calculated in relation to a reference period of 8 hours time-weighted average (unless otherwise specified)

### Human health values

| Relevant DNELs and other threshold levels |  |  |  |  |
| --- | --- | --- | --- | --- |
| Endpoint | Threshold level | Protection goal, route of exposure | Used in | Exposure time |
| DNEL | 8.22 mg/m³ | human, inhalatory | worker (industry) | chronic - systemic effects |
| DNEL | 2.33 mg/kg | human, dermal | worker (industry) | chronic - systemic effects |

| Relevant DNELs and other threshold levels |  |  |  |  |
| --- | --- | --- | --- | --- |
| Endpoint | Threshold level | Protection goal, route of exposure | Used in | Exposure time |
|  | bw/day |  |  |  |

## Environmental values

| Relevant PNECs and other threshold levels |  |  |  |  |
| --- | --- | --- | --- | --- |
| Endpoint | Threshold level | Organism | Environmental compartment | Exposure time |
| PNEC | 3.2 µg/l | aquatic organisms | freshwater | short-term (single instance) |
| PNEC | 0.32 µg/l | aquatic organisms | marine water | short-term (single instance) |
| PNEC | 0.219 mg/kg | aquatic organisms | freshwater sediment | short-term (single instance) |
| PNEC | 21.9 µg/kg | aquatic organisms | marine sediment | short-term (single instance) |
| PNEC | 41.9 µg/kg | terrestrial organisms | soil | short-term (single instance) |

## Exposure controls

### Appropriate engineering controls

General ventilation.

### Individual protection measures (personal protective equipment) Eye/face protection

Wear eye/face protection.

### Skin protection

- Hand protection

Wear protective gloves.

### Other protection measures

Take recovery periods for skin regeneration. Preventive skin protection (barrier creams/ointments) is recommended. Wash hands thoroughly after handling.

### Respiratory protection

Particulate filter device (EN 143).

### Environmental exposure controls

Use appropriate container to avoid environmental contamination.  Keep away from drains, surface and ground water.

# SECTION 9: Physical and chemical properties

## Information on basic physical and chemical properties

| Physical state | solid |
| --- | --- |
| Colour | not determined |
| Odour | characteristic |
| Melting point/freezing point | 283 °C |
| Boiling point or initial boiling point and boiling range | 651 °C |
| Flammability | this material is combustible, but will not ignite readily |

| Lower and upper explosion limit | not relevant (solid) |
| --- | --- |
| Flash point | >230 °C |
| Auto-ignition temperature | not determined |
| Decomposition temperature | not relevant |
| pH (value) | not applicable |
| Kinematic viscosity | not relevant |

### Solubility(ies)

| Water solubility | 350,000 mg/l at 20 °C |
| --- | --- |

### Partition coefficient

| Partition coefficient n-octanol/water (log value) | -1.28 (ECHA) |
| --- | --- |

| Vapour pressure | 0 Pa at 25 °C |
| --- | --- |

### Density and/or relative density

| Density | 1.05 g/cm³ |
| --- | --- |
| Relative vapour density | not relevant (solid) |

| Particle characteristics | no data available |
| --- | --- |

## Other information

| Information with regard to physical hazard classes | hazard classes acc. to GHS (physical hazards): not relevant |
| --- | --- |

### Other safety characteristics

| Solid content | 100 % |
| --- | --- |

# SECTION 10: Stability and reactivity

## Reactivity

Concerning incompatibility: see below "Conditions to avoid" and "Incompatible materials".

## Chemical stability

See below "Conditions to avoid".

## Possibility of hazardous reactions

No known hazardous reactions.

## Conditions to avoid

There are no specific conditions known which have to be avoided.

### Hints to prevent fire or explosion

The product in the delivered form is not dust explosion capable; the enrichment of fine dust however leads to the danger of dust explosion.

## Incompatible materials

Oxidisers

## Hazardous decomposition products

Reasonably anticipated hazardous decomposition products produced as a result of use, storage, spill and heating are not known. Hazardous combustion products: see section 5.

# SECTION 11: Toxicological information

## Information on toxicological effects Classification acc. to GHS

### Acute toxicity

Shall not be classified as acutely toxic.

### Skin corrosion/irritation

Causes skin irritation.

### Serious eye damage/eye irritation

Causes serious eye irritation.

### Respiratory or skin sensitisation

May cause an allergic skin reaction.

### Germ cell mutagenicity

Shall not be classified as germ cell mutagenic.

### Carcinogenicity

Shall not be classified as carcinogenic.

### Reproductive toxicity

Shall not be classified as a reproductive toxicant.

### Specific target organ toxicity - single exposure

Shall not be classified as a specific target organ toxicant (single exposure).

### Specific target organ toxicity - repeated exposure

Shall not be classified as a specific target organ toxicant (repeated exposure).

### Aspiration hazard

Shall not be classified as presenting an aspiration hazard.

## Information on other hazards

There is no additional information.

# SECTION 12: Ecological information

## Toxicity

Harmful to aquatic life with long lasting effects.

## Persistence and degradability

### Biodegradation

The substance is readily biodegradable.

## Bioaccumulative potential

Data are not available.

| n-octanol/water (log KOW) | -1.28 (ECHA) |
| --- | --- |

## Mobility in soil

Data are not available.

## Results of PBT and vPvB assessment

According to the results of its assessment, this substance is not a PBT or a vPvB.

## Endocrine disrupting properties

Information on this property is not available.

## Other adverse effects

Data are not available.

SECTION 13: Disposal considerations

## Waste treatment methods

### Sewage disposal-relevant information

Do not empty into drains. Avoid release to the environment. Refer to special instructions/safety data sheets.

### Waste treatment of containers/packagings

Completely emptied packages can be recycled. Handle contaminated packages in the same way as the substance itself.

## Remarks

Please consider the relevant national or regional provisions. Waste shall be separated into the categories that can be handled separately by the local or national waste management facilities.

SECTION 14: Transport information

  - UN number or ID number	not subject to transport regulations

  - UN proper shipping name	not relevant

## Transport hazard class(es)	none

  - Packing group	not assigned

  - Environmental hazards	non-environmentally hazardous acc. to the dan-gerous goods regulations

## Special precautions for user

There is no additional information.

## Maritime transport in bulk according to IMO instruments

The cargo is not intended to be carried in bulk.

Information for each of the UN Model Regulations

## International Maritime Dangerous Goods Code (IMDG) - Additional information

Not subject to IMDG.

## International Civil Aviation Organization (ICAO-IATA/DGR) - Additional information

Not subject to ICAO-IATA.

SECTION 15: Regulatory information

## Safety, health and environmental regulations/legislation specific for the substance or mixture

  - Chemical safety assessment

No Chemical Safety Assessment has been carried out for this substance.

# SECTION 16: Other information

## Indication of changes (revised safety data sheet)

| Section | Former entry (text/value) | Actual entry (text/value) | Safety-relevant |
| --- | --- | --- | --- |
| 1.1 | Identification of the substance: 1-Propanaminium, 3-amino-N-(carboxymethyl)-N,N-dimethyl-, N-coco acyl derivs., hydroxides, in-ner salts | Identification of the substance: Coco amidopropyl betaine | yes |
| 1.1 |  | Alternative name(s): 1-Propanaminium, 3-amino-N-(carboxymethyl)-N,N-dimethyl-, N-coco acyl derivs., hydroxides, in-ner salts | yes |
| 3.1 | Name of substance: 1-Propanaminium, 3-amino-N-(carboxymethyl)-N,N-dimethyl-, N-coco acyl derivs., hydroxides, in-ner salts | Name of substance: Coco amidopropyl betaine | yes |

## Abbreviations and acronyms

| Abbr. | Descriptions of used abbreviations |
| --- | --- |
| ADR | Accord relatif au transport international des marchandises dangereuses par route (Agreement concern-ing the International Carriage of Dangerous Goods by Road) |
| CAS | Chemical Abstracts Service (service that maintains the most comprehensive list of chemical substances) |
| Ceiling-C | Ceiling value |
| DGR | Dangerous Goods Regulations (see IATA/DGR) |
| DNEL | Derived No-Effect Level |
| EC No | The EC Inventory (EINECS, ELINCS and the NLP-list) is the source for the seven-digit EC number, an identi-fier of substances commercially available within the EU (European Union) |
| EH40/2005 | EH40/2005 Workplace exposure limits (http://www.nationalarchives.gov.uk/doc/open-government-li-cence/) |
| EINECS | European Inventory of Existing Commercial Chemical Substances |
| ELINCS | European List of Notified Chemical Substances |
| GHS | "Globally Harmonized System of Classification and Labelling of Chemicals" developed by the United Na-tions |
| IATA | International Air Transport Association |
| IATA/DGR | Dangerous Goods Regulations (DGR) for the air transport (IATA) |
| ICAO | International Civil Aviation Organization |
| IMDG | International Maritime Dangerous Goods Code |
| NLP | No-Longer Polymer |
| PBT | Persistent, Bioaccumulative and Toxic |
| PNEC | Predicted No-Effect Concentration |
| ppm | Parts per million |
| REACH | Registration, Evaluation, Authorisation and Restriction of Chemicals |
| RID | Règlement concernant le transport International ferroviaire des marchandises Dangereuses (Regulations concerning the International carriage of Dangerous goods by Rail) |
| STEL | Short-term exposure limit |
| TWA | Time-weighted average |

| Abbr. | Descriptions of used abbreviations |
| --- | --- |
| vPvB | Very Persistent and very Bioaccumulative |
| WEL | Workplace exposure limit |

## Key literature references and sources for data

Agreement concerning the International Carriage of Dangerous Goods by Road (ADR). Regulations concerning the In-ternational Carriage of Dangerous Goods by Rail (RID). International Maritime Dangerous Goods Code (IMDG). Dan-gerous Goods Regulations (DGR) for the air transport (IATA).

## List of relevant phrases (code and full text as stated in section 2 and 3)

| Code | Text |
| --- | --- |
| H315 | Causes skin irritation. |
| H317 | May cause an allergic skin reaction. |
| H319 | Causes serious eye irritation. |
| H412 | Harmful to aquatic life with long lasting effects. |

## Disclaimer

This information is based upon the present state of our knowledge. This SDS has been compiled and is solely intended for this product.