# SAFETY DATA SHEET (SDS)

| 1. Product Description |  |
| --- | --- |
| Product Name | Avocado Oil Extra Virgin |
| Product Number | All Avocado Oil items |
| Brand | All |
| Supplier | Formulator County 2300 kathryn ln plano tx 75025 sales@deoorganic.com |
| Emergency Phone Number | Chemtrec (800) 424-9300 |
| Use of substance | Skin care ingredient |

| 2. HAZARD IDENTIFICATION |  |
| --- | --- |
| Description | Appearance: Deep green Physical State: Liquid Odor: Slight Vegetable oil |
| Handling | If smoking occurs from oil usage, reduce or remove from heat. Spontaneous combustion (fire) may result from oil soaked materials such as rags, steel wool, paper and clothing. Place soaked materials in a sealed, metal container to prevent this. If smoking occurs from oil usage, reduce or remove from heat. This product is NOT classified as hazardous according to CFR 1910, amended to conform to the United Nations’ Globally Harmonized System of Classification and Labeling of Chemicals (OSHA / GHS); SOR/88-66, the Canadian Controlled Products Regulations (CPR); and/or NOM-002-SCT-2003 (Mexico). However, vegetable oil (in mist form) is known to be listed as an OSHA 29 CFR 1910.1000 Air Contaminant. Occupational exposure limits are subsequently provided in section 8 of this SDS. |
| OSHA Hazards | Not a dangerous substance according to GHS. GHS Pictogram: Exclamation Mark Signal Word: Warning |
| Other Information | May be harmful if inhaled. May cause respiratory tract irritation. May cause slight eye irritation. |

| 3. COMPOSITION/INFORMATION ON INGREDIENTS |  |
| --- | --- |
| Synonyms | Avocado Oil @100% |
| GRAS status | All ingredients are FDA GRAS. |
| Hazard status | No food ingredients are hazardous according to OSHA criteria. |

| 4. FIRST AID MEASURES |  |
| --- | --- |
| Inhalation | Move to fresh air in case of accidental inhalation of vapors or decomposition products. |
| Skin contact | Wash off with soap and plenty of water. |
| Eye contact | Flush eyes with water of special eyewash solution. |
| If swallowed | Ingestion of edible vegetable oil is nontoxic and should pass through the system. |

| 5. FIREFIGHTING MEASURES |  |
| --- | --- |
| Conditions of flammability | Materials may pose fire hazard. |
| Suitable extinguishing media | Alcohol resistant foam, dry chemical or carbon dioxide. Use extinguishing measures that are appropriate to local circumstances and the surrounding area. |
| Special protective equipment for firefighters | Wear self-contained breathing apparatus for firefighting if necessary. |
| Specific hazards arising from the Chemical Oil | Risk of ignition. Rags and other materials containing this product may heat up and spontaneously ignite if exposed to air. Store wiping rags and similar materials in metal cans with tightly fitting lids. Cool closed containers exposed to fire with water spray. Avoid hot oil; if smoking occurs during application reduce or remove from heat. |
| Hazardous combustion products | Hazardous decomposition products formed under fire conditions. Nature of decomposition products not known. |
| Extinguishing Media | Foam, dry chemical powder, carbon dioxide |

| 6. ACCIDENTAL RELEASE MEASURES |  |
| --- | --- |
| Personal Protection | Avoid breathing vapors, mist or gas. Recommend exhaust fans over grills and deep frying. |
| Environmental Precautions | Prevent further leakage or spillage. Do not allow product to reach soil, sewage or any water sources. Dispose per local, state, and federal regulations. |
| Methods and materials for containment and cleaning up | Keep in suitable, closed containers for disposal. Dispose of rags used in clean up. Remember oil soaked rags or partially cleaned materials may spontaneously combust. |

| 7. HANDLING AND STORAGE |  |
| --- | --- |
| Conditions for safe storage | Ensure adequate dry, well ventilated storage area. Protect against sunlight. Clean up any spillage to avoid accidents immediately. |
| Combustible conditions | Keep away from open flames, hot surfaces and sources of ignition. |

| 8. EXPOSURE CONTROLS/PERSONAL PROTECTION |  |
| --- | --- |
| Occupational exposure limit values | Contains no substances with occupational exposure limited values. |
| Respiratory protection | Respiratory protection not required. For nuisance exposures use type OV/AG or type ABEK respiratory cartridges. |
| Hand protection | Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique to avoid skin contact with this product. Dispose of contaminated gloves after use in accordance with applicable laws and good laboratory practices. Wash and dry hands. |

| Eye protection | Use equipment for eye protection tested and approved under appropriate government standards such as NIOSH (USA) and EN 166 (EU). Oil in eye must be flushed with water or special first aid eye wash continuously. |
| --- | --- |
| Skin and Body protection | Impervious clothing. The type of protective equipment must be selected according to the concentration and amount of the dangerous substance at the specific workplace. Deep frying or grill operator must use nonabsorbent apron when dealing with hot oil. |
| Hygiene measures | Good industrial hygiene practices. |

| 9. PHYSICAL AND CHEMICAL PROPERTIES |  |
| --- | --- |
| Form | Clear, liquid |
| Color | Deep green |
| pH | 4-6 |
| Specific gravity | @0.910 - 0.925 (H20 = 1) |
| Melt point/freezing point |  |
| Smoke point | >439ºF |
| Flash point | >644ºF |
| Ignition temperature | >680ºF |
| Auto-ignition | Not auto flammable |
| Lower explosion limit | No data available |
| Upper explosion limit | No data available |
| Vapor pressure | No data available |
| Water solubility | Insoluble |
| Partition coefficient | No data available |
| Relative vapor | No data available |
| Odor/Flavor | Slight bland odor |
| Odor threshold | No data available |
| Evaporation rate | No data available |

| 10. STABILITY AND REACTIVITY |  |
| --- | --- |
| Chemical stability | Stable under recommended storage conditions. Cooking and frying temperature >439oF, oil will smoke. Reduce or remove from heat. |
| Possibility of hazardous reaction | None known. |
| Conditions to avoid | Keep away from open flames, hot surfaces and sources of ignition. |
| Materials to avoid | Strong oxidizing agents. |
| Hazardous decomposition products | Hazardous decomposition products formed under fire conditions – nature of decomposition not known. Other decomposition products – no data available Rags used for clean-up (clean or contaminated) can combust if conditions are adequate. Keep in safe place or dispose of rags after usage in an enclosed metal container. |

| 11. TOXICOLOGICAL INFORMATION |  |
| --- | --- |
| Acute toxicity | Oral: LD50 no data available. Inhalation: LC50 no data available. Dermal: LD50 no data available. |
| Skin corrosion | No data available. |
| Serious eye damage | No data available. |
| Respiratory or skin sensitization | No data available. |
| Carcinogenicity IARC | No component of this product present at levels greater than or equal to 0.1% is identified as probable, possible, or confirmed carcinogenic by IARC. |
| Carcinogenicity ACGHI | No component of this product present at levels greater than or equal to 0.1% is identified as probable, possible, or confirmed carcinogenic by ACGIH. |
| Carcinogenicity NTP | No component of this product present at levels greater than or equal to 0.1% is identified as probable, possible, or confirmed carcinogenic by NTP. |
| Carcinogenicity OSHA | No component of this product present at levels greater than or equal to 0.1% is identified as probable, possible, or confirmed carcinogenic by OSHA. |
| Reproductive toxicity | No data available. |
| Teratogenicity | No data available. |
| Specific target organ toxicity | No data available. |
| Aspiration hazard | No data available. |
| Potential health effects | Inhalation: May be harmful if inhaled. Ingestion: The material is a non hazardous food ingredient. Skin: May be harmful if absorbed through skin. Eyes: May cause eye irritation. |
| Signs and symptoms of exposure | No data available. |
| Synergistic effects | No data available. |
| Additional information | No data available. |

| 12. Ecological Information |  |
| --- | --- |
| Toxicity | No data available. |
| Persistence and degradability | Readily biodegradable. |
| Bio accumulative potential | Not applicable. |
| Mobility | No data available. |
| PBT and vPVB assessment | No data available. |
| Other adverse effects | No data available. |

| 13. DISPOSAL CONSIDERATIONS |  |
| --- | --- |
| Recycle | Whenever possible, as rules and regulations allow, please recycle or manage materials to minimize waste. |
| Waste disposal methods | Oil soaked materials may spontaneously combust and should be properly managed to avoid ignition and heat sources or oxygen rich environments. Collect and store soaked materials in closed, metal containers. |
| Contaminated packaging | Containers should be decontaminated and taken for local recycling, recovery or waste disposal facility. Follow local, state and federal guidelines. |

| 14. TRANSPORT INFORMATION |  |
| --- | --- |
| DOT (US), Canada (TDG), Mexico (MEX) | Not regulated; Not dangerous goods. |
| IMDG | Not regulated; Not dangerous goods. |
| ICAO | Not regulated; Not dangerous goods. |
| IATA | Not regulated; Not dangerous goods. |

15. REGULATORY INFORMATION

International Inventories: The components of the product are reported in the following inventories:

| Chemical Name | TSCA | DSL | NDSL | EINECS | ELINCS | AICS | ENCS ISHL | CHINA | PICCS | KECL | NZLoC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Avocado Oil | Yes | Yes | No | No | No | No | No | Yes | No | Yes | Yes |

Legend:

| TSCA | Toxic Substances Control Act Section 8(b) Inventory |
| --- | --- |
| DSL | Domestic Substance List (Canada) |
| NDSL | Non Domestic Substance List (Canada) |
| EINECS | European Inventory of existing commercial chemical substances |
| ELINCS | European List of Notified Chemical Substances |
| AICS | Australian Inventory of Chemical Substances |
| ENCE | Existing and new Chemical Substances (Japan) |
| ISHL | Industrial Health and Safety Law (Japan) |
| CHINA | Chinese Inventory of Existing Chemical Substances (China) |
| PICCS | Inventory of Chemicals and Chemical Substances ((Philippines) |
| KECL | Korean Existing and Evaluated Chemical Substances (Korea) |
| NZloC | New Zealand Inventory of Chemicals (New Zealand) |

| USA | OSHA Hazards No known OSHA hazards SARA 302 Components SARA 302: No chemicals in this material are subject to the reporting requirements of SARA Title III, Section 302. SARA 313 Components SARA 313: This material does not contain any chemical components with known CAS numbers that exceed the threshold (De Minimis) reporting levels established by SARA Title III, Section 313. SARA 311/312 Hazards No SARA Hazards Massachusetts Right To Know Components No components are subject to the Massachusetts Right to Know Act. Pennsylvania & New Jersey Right To Know Components Avocado oil extractives and their physically modified derivatives. Consists primarily of glycerides of the fatty acids oleic, Clean Air Act, Section 112 Hazardous Air pollutants (HAPs) (see 40 CFR 61) ..Product is not known to contain HAPS. California Prop. 65 Components This product does not contain any chemicals known to State of California to cause cancer, birth defects, or any other Reproductive harm. |
| --- | --- |
| Canada | Domestic transport regulations (Canada) WHMIS Product Classification Not a WHMIS controlled product. 15. REGULATORY INFORMATION WHMIS Ingredient Disclosure List IDL No known component is listed on the WHMIS ingredients disclosure list. (NPRI) Canadian National Pollutant Release Inventory No known component is listed on NPRI. This product has been classified in accordance with the hazard criteria of the Controlled Products Regulations (CPR) and the SDS contains all the information required by the CPR. |
| Mexico | Mexico - Grade Slight risk, Grade 1 |

| 16. OTHER INFORMATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| NFPA | Health Hazard | 0 | Flammability 1 | Reactivity 0 |
| HMIS | Health Hazard | 0 | Flammability 1 | Physical Hazard 0 |

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at the date of its publication. The information given is designed only as a guidance for safe handling, use, processing, storage, transportation, disposal and release and is not to be considered a warranty or quality specification either expressed or implied. The information is related only to the specific material designated and may not be valid for such material used in combination with any other materials or in any process, unless specified in the text.

The information in this SDS was obtained from current and reliable sources. However, the data is provided without any warranty, express or implied, regarding its correctness or accuracy. Since the conditions of use handling, storage and disposal of this product are beyond our control; it is the user’s responsibility both to determine safe conditions for use of this product and to assume liability for loss, injury, damage, or exposure due to improper use of this product.

Key for Abbreviations

| ACGIH TLV | American Conference of Governmental Industrial Hygienists Threshold Limit Values |
| --- | --- |
| AICS | Australian Inventory of Chemical Substances (Australia) |
| CAS | Chemical Abstract Service |
| CHINA | Chinese Inventory of Existing Chemical Substances (China) |
| DOT | U.S. Department of Transportation |
| DSL | Domestic Substance List (Canada) |
| EINECS | European Inventory of Existing Commercial Chemical Substances (EU) |
| ELINCS | European List of Notified Chemical Substances (EU) |
| ENCS | Existing and New Chemical Substances (Japan) / ISHL |
| GHS | Globally Harmonized System of Classification and Labeling of Chemicals |
| IATA | International Air Transport Association Dangerous Goods Regulations |
| ICL | In Commerce List (Canada) |
| IMDG | International Maritime Dangerous Goods Code |
| IMO | International Maritime Organization |
| KECL | Korean Existing and Evaluated Chemical Substances (Korea) |
| LC50 | Lethal concentration that produces fatalities in 50% of a given test population |
| LD50 | Median lethal dose of a given test population |
| MEX | NOM |
| MEXICO | México Occupational Exposure Limits |
| NDSL | Non Domestic Substances List (Canada) |
| NFPA | National Fire Protection Association |
| NIOSH | National Institute of Occupational Safety and Health |
| NZIoC | New Zealand Inventory of Chemicals (New Zealand) |
| OSHA | Occupational Safety & Health Administration |
| OSHA PEL | Occupational Safety and Health Administration Permissible Exposure Limits |
| PICCS | Inventory of Chemicals and Chemical Substances (Philippines) |
| STOT | Specific Target Organ Toxicity |
| TDG | Transportation of Dangerous Goods (Transport Canada) |
| TSCA | Toxic Substances Control Act, Section 8(b) Inventory (USA) |
| TWA | Time Weighted Average: Average concentration that should not be exceeded during a work day (usually 8 |